◎PROFILE 1

PROFILE 2

◎Material

Applicable Material:Galvanized Sheet Coil & Cold Rolled Steel

Materail Thickness:0.8mm；Feeding Width:400mm;

Material Yield Strength:235-345Mpa；

Effective Width Tolerance:±1mm；

The Wave Height Tolerance:±1mm；

The Length Of Tolerance:±1mm(6m)；

◎Work Flow

Hydraulic Uncoiler→Feeding Guide Device→Leveling Device→Hydraulic Punching Device→Hydraulic Pre-Cutting Device→Main Forming System(Casstte Change Profiles)→Hydraulic Post Cutting Device→Exit Rack

◎The Shared Device

1.Hydraulic Uncoiler                6.Body Frame H450 Steel

2.Leveling Device                  7.Exit Rack

3.Hydraulic Pre-Punching Device      8.Motor

4.Hydraulic Pre-Cutter Device         9.PLC Control System

5.Post Hydraulic Cutter Device        10.Hydraulic Station

◎Then Change Device

1.Roll Forming Part

2.Post Cutting Mould

◎5T-500 Hydraulic Uncoiler:One Set

Hydraulic Control Steel Coil Inner Bore Shrinkage And Stop

Max. Feeding Width:600mm

Coil ID Range:508+/-30mm

Max Capacity:5 T

◎Leveling Device:One Set

With Feeding Material Guide

Side Wall Thickness: Q235 t18mm

Up Three Down Four Totally Seven Shafts Leveling Device

Body Frame Made From H450 Type Steel By Welding

Shafts Make From GCr15 Steel,Diameter=φ95mm,High Frequency Heat Treatment

◎Hydraulic Pre Punching & Cutting Device:One Set

Pre-Punching & Cutting And Post-Cutting,Use The Same Hydraulic System

Hydraulic Motor:11KW,Hydraulic Pressure Range:0-16Mpa

Punching & Cutting Tool Material:Cr12MoV(=D2 Steel),Heat Treatment To HRC58-62

5 Hydraulic Cylinders Punching System For Punching Both On Web And Flange

The Punching On The Web And Flange Will Be Done Through PLC Programme Automatic;

Punching Holes Show:

A Cylinder:To Punching The 7mm*17mm Oval Holes(One Punching Mould)；

B Cylinder:Punching The 5.5*10mm(One Punching Mould)；

C Cylinder:Punching The 6mm*14mm(One Punching Mould)；

D Cylinder:Punching The Dimple Hole(One Punching Mould)；

E Cylinder:Use The E Cylinder To Punching Left And Right Notch,Then Use The Post Cutting Device To Cutting The 8mm Width Material.

Hydraulic Pre-Cutting Device:One Set

Profile 1 & Profile 2 All Use 400mm Feeding Width,But When Change The Roll forming Part need Use Hydraulic Pre-cutting Device To Cut The Plate,So I add This Cutting For you.

Hydraulic Station With Air Cooling Device:One Set

◎Roll Forming Part:One Set

Body Frame Made From H450 Type Steel By Welding;

Side Wall Thickness:Q235 t18mm;

Rollers Make From 45# Steel,Precision Work,Hard Chrome Plating;

Shafts Diameter=ф75,Make From 45#,Quenching And Tempering;

Gear/Sprocket Driving,18-Steps To Form Profile1 & 18 Steps To Form Profile 2;

Servo Motor=11KW , Frequency Speed Control;

Speed Redcucer Motor:K Series;

Sliding Block:130*140mm;All The Screw Bolts With Grade 8.8;

Real Forming Speed:22m/min;

◎Post Hydraulic Cutting Device

Post To Cut, Stop To Cutting,Three Pieces Type Of Cutting Blade Design;

Hydraulic Motor:11KW;Cutting pressure: 0-16Mpa;

Cutting Tool Material:Cr12MoV(= SKD11 With At Least One Million Times Of Cutting Life),Heat Treatment To HRC58-62 Degree;

The Cutting Power Is Provided By The Main Engine Hydraulic Station;

◎PLC Control System:One Set

Control The Quantity And Cutting Length Automatically Input The Production

Data(ProductionBatch,pcs,Length,etc.)On The Touch Screen,It Can Finish The

Production Automatically.Combined With: PLC, Inverter, Touch Screen, Encoder

Switch Bottom, Indicate Light, Power Supply, Intermediate Relay, AC Contactor;

PLC:Schneider(France Brand);

Inverter:Schneider(France Brand);

7-Inch Color Touch Screen:Schneider(France Brand);

Encoder:Omron (Japan Brand);

Thermal Relay:Schneider(France Brand);

Air Switch:LG-LS(Korea Brand);

Cut-to-Length Tolerance≤±1 mm;

Control Voltage 24V;

Chinese & English Language

◎4 Meter Exit Rack:One Set

◎Coupler Hydraulic Punching Device:Two Sets

◎Tools And Spare Parts:One Set

Open Wrench 24x22, 22x19, 19x17, 17x14

A Hex Wrench (Size 1 to 10)

A Vise,A Cross And A Flat-Head Screwdriver, A Feeler,A Iron A Hammer, A Tape Measure

Spare Parts:

1.Drive Chain x 1

2.Drive Bearing x 2

3.Limite Switch x 1

◎Sample Show

◎Price

| NO. | Item Name | Quality | Delivery Time |
| --- | --- | --- | --- |
| 1 | 5T-500 Hydraulic Uncoiler | 1 | 75 days |
| 2 | Leveling Device | 1 |  |
| 3 | Hydraulic Pre-Punching Device | 1 |  |
| 4 | Hydraulic Pre-Cutting Device | 1 |  |
| 5 | Roll Forming Part | 2 |  |
| 6 | Post Hydraulic Universal Cutting Device | 1 |  |
| 7 | PLC Control System | 1 |  |
| 8 | Exit Rack | 3 |  |
| 9 | Coupler Hydraulic Punching Device | 2 |  |
| 10 | Tools And Spare Parts | 1 |  |